Instruction Manual

IMPORTANT

- Before using this battery charger, please read this manual carefully and make sure you fully understand all the instructions and safety rules contained in this manual; and keep it for your future reference.

- To avoid risk of fire, electric shock and other hazards when using this charger, please follow the instructions strictly.

Product Illustrations

Charger Base

Front; Back

Dual USB Output

LED Indicator

Interchangeable Adaptor Plate Receiving Bay

7-Pin Metal Contact

AC Power Cord Port

Adapter Plate Lock Button

Interchangeable Adapter Plate

Charging Contact

Power Input Metal Contact

7-Pin Metal Contact

Front / Back

AC Power Cord x 1

There are 5 types of AC Power Cord for choice (Optional):

Australian  American  British  European  Japanese

Cord Length:	1100 mm

Output Plug:	IEC C5

Output:	USB × 2 (DC 5V / 2.1A max. for each port)

To Connect the Power Cord to the Charger Base

To Install the Adapter Plate onto the Charger Base

PUSH

Place the adapter plate onto the charger base from the top into the slots.

Push the charger plate fully in the direction as shown in the above diagram.

The plate is fully inserted into the charger base only when you hear a “click” sound.

To Remove the Adapter Plate from the Charger Base

Push Button (at bottom)

PUSH

Press and hold the release button to unlock the charger plate from the charger base.

Push the charger plate in the direction as shown in the above diagram to remove the adapter plate from the charger base.

To Install the Power Tools Battery into the Charger

PUSH

PRESS

Install the battery in the direction as shown in the above two diagrams.

Diagram A and B is the different direction to install battery, and you should install as your battery’s model.

The Dual USB Output Port provide 5V power source for many popular mobile devices including mobile phones, tablet PCs, mobile router, and external batteries, etc.

Dual USB (Type A) Output

(DC 5V / 2.1A max. for each port)

Specifications

| Model | ACMG |
| --- | --- |
| Input | 100 – 240V AC, 50-60Hz, 1.2A, 70W |
| Output | DC 21.6V – 36V, 1 - 2A |
|  | USB x 2 (DC 5V / 2.1A max. for each port) |
| Weight | Approx. 710g |
| Protection | Short Circuit Protection Reverse Protection Over Temperature Protection Max. 4 Hrs Charging Limit Protection Low Voltage Slow Charge Safety Protection |
| Dimensions | 185 (L) x 160 (W) x 80 (H) mm |

Operation InstructionS

- Connect the supplied power cord to the Power Tools Battery Charger Base and plug the AC plug into an AC wall outlet.

- The LED indicator in the front of the charger base will start flashing GREEN when the charger is ready.

- When the battery pack is placed onto the charger plate, the LED indicator will turn steady RED when the battery is being charging.

- When the battery pack is damaged, or there is a connection error, or when the battery charger is overheated, the LED indicator will start flashing RED.

- When the battery pack is fully charged, the LED indicator turns steady GREEN.

- After the battery pack is fully charged you can disconnect the charger from AC power outlet, then release the battery pack from the charger plate and the battery is now ready for use.

SAFETY INSTRUCTIONS AND WARNING

1.	Do NOT use this ACMG charger for charging battery packs other than the specified rechargeable ones. Improper use of the charger or attempting to charge non-rechargeable batteries may cause fire, electrocution or other hazards.

2.	In accordance with Section 7.12 of IEC 60335-1:2010, this charger is not intended for use by persons (including children) with reduced physical, sensory or mental capabilities, or lack of experience and knowledge, unless they have been given supervision or instruction concerning use of the charger by a person responsible for their safety.

3.	The charger is designed for charging the following battery packs:

- 15-cell Ni-Cd/Ni-MH in series, with max. capacity of 10Ah.

- 5-cell Li-ion in series, with max. capacity of 10Ah.

4.	The charger should only be used in-door under adequate ventilation (ventilation slots must under no circumstances be covered or blocked); and it must be connected to an earthed socket-outlet at any time. There is danger of short-circuit or electrocution should it be used in humid rooms or outdoors.

5.	Do not attempt to disassemble or modify this unit. All inspections and repair works should be carried out by qualified service personnel only. No liability will be assumed in the case of non-compliance of safety instructions and usage.

6.	Only use the AC power cord comes with the charger or the one specified by manufacturer. The power cord must be safely connected.

7.	If the power cord is damaged, it must be replaced by the manufacturer, its service agent or qualified person in order to avoid any hazard.

8.	Unplug the charger from the AC outlet before connecting / disconnecting the battery pack; and when it is no longer in use. Otherwise there is danger of electrocution.

9.	When the metal contacts of the charger get dirty, unplug the charger from the AC outlet and then clean them only with dry soft cloth. Otherwise there is danger of electrocution.

10.	During charging, the temperature of the charger will rise. This is normal and not a malfunction.

11.	Stop charging the battery immediately if charging does not complete in reasonable time.

12.	If abnormal smell is detected and case deformity occurred during use, turn off the power supply immediately and unplug the power cable from socket to avoid possible damage.

13.	Store the charger in a cool and dry place when not in use.

14.	Do not expose the charger to direct sunlight and other heat sources (e.g. radiators and other electrical devices, and chimney, etc.), moisture, water, sparks, fire, corrosive materials or explosive gas; and to locations where liquids (e.g. condensation and water coming from leaking roofs) or objects can get into the charger.

15.	Do not dispose this product together with household waste. The product and its accessories must be disposed in strict accordance with local environmental protection laws and regulations, e.g. the WEEE Directive (2002/96/EC).

16.	Keep the charger out of the reach of infants and children. Children must not play unsupervised near the product and its connections.