SPECIFICATION LEX1483

R E V：         A

TYPE：   SPS 85W SERIES

SPEC：      14.5V—18.5V

- 1. ElECTRICAL

  - Input Characteristics:

- 1.1.1 Rated Voltage

It is normal for 100-240Vac input AC voltage

- 1.1.2 Input Voltage range

The adapter shall operate from 90Vac to 264Vac and no range switching is necessary or possible.

- 1.1.3 Rated Frequency

The frequency of adapter was changed when it work in different model.

- 1.1.4 Frequency range

The adapter shall operate with an input frequency from 47Hz to 63Hz

- 1.1.5 Steady AC Current

Maximum steady state input current is less than 1.8A rms at 100Vac input and maximum load.

- 1.1.6 Inrush Current

Maximum inrush current shall be less than 40A at 240Vac input.

- 1.1.7 Power Efficiency

80% Min. measured at 100Vac input voltage, maximum load and include the DC cable loss.

  - Output Characteristics

- 1.2.1 Rated Voltage

The rated output voltage is specified from 14.5V to 18.5V

- 1.2.2 Voltage Range

The output voltage will be performed ±0.35V

- 1.2.3 Rated Power

This adapter is capable to support 85Watts continuously at all specified conditions.

- 1.2.4 Current

Maximum current is 5.8A when the output voltage is 14.5V.

Maximum current is 4.6A when the output voltage is 18.5V.

- 1.2.5 Output Ripple and Noise

Output ripple voltage is 200mV peak to Peak or less.

(Ripple voltage is specified in working cycle of this supply ).

Output noise voltage is 200mV peak to Peak or less.

(Noise voltage is specified in the switching noise of this supply ).

Measured methods:

T1. Performed by 20MHz bandwidth in oscilloscope.

T2. Applied 0.1uF ceramic capacitor and 10uF electrolytic capacitor across output

Connector terminals.

T3. Measured at the end of DC cable.

- 1.2.6 Stability

The adapter is unconditionally stable while operating within its normal operating specification.

- 1.2.7 Protection.

- 1.2.7.1 Short Circuit

The adapter is protected when a short happened between the output terminals and shall not result in a fire hazard, any damage to this adapter and will be normal operation automatically while the short is removed.

- 1.2.7.2 Over current protection

The power supply shall be protected when output current over each peak current For example when the output voltage is 18.5V,the power supply shall be protect when the output current over 7A.

      - Over voltage protection

The power supply shall be protected when output voltage over 33V.

- 2. Environmental

- 2.1 Temperature

- 2.1.1 Operating

The adapter is capable to operate from 0 to 45℃.

- 2.1.2 Non-Operating

The adapter is capable to operate from -20 to 80℃.

- 2.2 Humidity

- 2.2.1 Operating

The adapter is capable to operate from 10 to 90%RH

- 2.2.2 NON-Operating

The adapter is capable to operate from 5 to 90%RH

- 2.3 Cooling

The adapter is convention cooled only

- 2.4 Altitude

- 2.4.1 Operating

The adapter is capable to operate at 10,000 feet above sea level

- 2.4.2 Non-Operating

The adapter is capable be stored at 33,000 feet above sea level

- 2.5 Vibration

- 2.5.1 Operating

AC input voltage: Rated voltage

DC output load: Maximum

Acceleration: 1.0G

Orientation: Three mutually perpendicular axes X,Y,Z.

Frequency 10 to 300Hz at 0.3 decades/min.

Duration: 30 minutes per axis,15 minutes at a constant acceleration of 2.0G for resonant frequencies

- 2.5.2 Non-Operating(constant vibration)

Acceleration: 3.0G

Orientation: Three mutually perpendicular axes X,Y,Z.

Frequency 10 to 300Hz at 0.3 decades/min.

Duration: 30 minutes per axe, 15 minutes at a constant acceleration of 3.0G for resonant frequencies.

After the above testing, the adapter have to work normally and no any damage.

- 2.6 Drop(Non-operating)

The adapter is capable to survive a drop from 100cm height to rigid surface of concrete.

It shall be dropped once for each one of its six without any packaging after the completion of the drop test the adapter have to meet function within specifications.

No structural damage is allowed. Cosmetic damages such as small dents and scratches are acceptable.

- 2.7 Dielectric Withstand Voltage(HI-POT)

100% of line products of this adapter shall be applied 3000VAC or 4200VDC for a minimum of 1 minute between AC input terminals and output terminals. The cut off current is specified as 10mA.

- 2.8 Leakage Current

The measured reading is less than 0.10mA at 264 V AC,50Hz.

- 2.9 Insulation Resistance

The resistance is more than 100MΩ at apply 500 VDC between input terminals and Case.

The resistance is more than 100MΩ at apply 500 VDC between input terminals and output terminals.

3. Mechanical

- 3.1 Adapter drawing

The external dimensions of adapter is 101.00mm(Length)×69.00mm(Width)×29.00mm(Height)

- 3.2 Material

The case material is PC.Class 94V-0.

- 3.3 Color

The color is specified in WHITE.

- 3.4 DC Cord

- 3.4.1 Output cord :  8A WHITE 2.0M±20mm

- 3.4.2 Output plug :

- 3.4.3 Output polarity:

- Outline Dimension(unit:mm)

Testing

| Model No.: SPS 85W SERIES AC input range: 100Vac-240Vac |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NO. | No load voltage (VDC) | Rated voltage I=5.66A (VDC) | Ripple noise (mV) | Efficiency | Over current (A) | Short circuit | Leakage current test | Insulation resistance test | Hi pot test | Over Temperature |
| SPEC | 18.5V±0.5 | 18.5±0.5 | 300Mv Max | 80% Min | 7A Max | No Damage | ILK＜0.10mA | 500VDC 100MΩ Min | PRI-SEC:3.0KV AC PRI-CASE:3.0KV AC 10MA 1Min | 70℃±3 No Damage |
| 1 | 18.8 | 18.1 | OK | 80.3% | 6.2 | OK | OK | OK | OK | OK |
| 2 | 18.7 | 18.2 | OK | 80.3% | 6.1 | OK | OK | OK | OK | OK |
| 3 | 18.8 | 18.1 | OK | 80.4% | 6.2 | OK | OK | OK | OK | OK |
| 4 | 18.8 | 18.2 | OK | 80.3% | 6.1 | OK | OK | OK | OK | OK |
| 5 | 18.8 | 18.1 | OK | 80.5% | 6.2 | OK | OK | OK | OK | OK |
| 6 |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |